Mass Budget Assessment

OrbitGuard Assembly 3.2.2 — Thruster Configuration

Date:  11 May 2026  Status:  Preliminary — Pending Material Verification

1.  Methodology

Part volumes were computed directly from the SolidWorks STEP assembly (OrbitGuard_Assembly_3.2.2_Thruster.STEP) using the OpenCASCADE geometry kernel. The assembly hierarchy was traversed via the XCAF framework to identify 67 unique part definitions across 320 total instances. Volume properties were evaluated on each closed B-rep solid; mass was then derived by applying material densities assigned per part on the basis of part nomenclature. COTS items and parts with non-descriptive identifiers were assigned a default aluminium density of 2700 kg/m³ pending confirmation. All results should be treated as preliminary until material assignments are verified against design records.

2.  Results

The estimated total assembly mass is 45.2 kg. Mass is heavily concentrated in the structural panels, which together account for approximately 75% of the total. The breakdown by category is given in Table 1.

| Category | Total Mass (g) | % of Total |
| --- | --- | --- |
| Structural panels (6 face panels + shear panel) | 33,882 | 75.0% |
| Propulsion (2× BHT-350 T1 thrusters) | 4,240 | 9.4% |
| Brackets and secondary structure | 3,965 | 8.8% |
| Electronics / RFIM assembly | 752 | 1.7% |
| Fasteners (bolts, washers) | 116 | 0.3% |
| Other / unresolved COTS | 230 | 0.5% |
| TOTAL | 45,185 | 100% |

Table 2 lists the ten highest-mass line items, sorted by total contribution. Panel geometry is modelled as solid aluminium; actual panel construction (likely sandwich with CFRP face sheets) would reduce the panel mass contribution significantly — see Section 3.

| Part | Qty | Vol/unit (cm³) | Density (kg/m³) | Mass/unit (g) | Total (g) |
| --- | --- | --- | --- | --- | --- |
| Z− Panel | 1 | 5132 | 2700* | 13,857 | 13,857 |
| Z+ Panel | 1 | 2081 | 2700* | 5,617 | 5,617 |
| BHT-350 T1 thruster | 2 | 785 | 2700* | 2,120 | 4,240 |
| X+ Panel | 1 | 1154 | 2700* | 3,117 | 3,117 |
| Y− Panel | 1 | 1117 | 2700* | 3,017 | 3,017 |
| Y+ Panel | 1 | 1111 | 2700* | 3,001 | 3,001 |
| X− Panel | 1 | 1110 | 2700* | 2,998 | 2,998 |
| Shear Panel | 1 | 843 | 2700* | 2,275 | 2,275 |
| Main Thruster Bracket | 2 | 268 | 2700 | 723 | 1,446 |
| RS-00000821 | 1 | 366 | 2700* | 988 | 988 |

* Density assumed from part name; pending material confirmation.

3.  Recommendations

3.1  Assign materials in SolidWorks

Material properties are not exported to STEP AP214, so the analysis above relies on density values inferred from part names. The most direct remedy is to assign the correct material to each part file in SolidWorks (Edit Material in the part document). Once materials are assigned, SolidWorks' built-in Mass Properties tool (Evaluate → Mass Properties at assembly level) will report accurate masses directly from the model, without requiring any post-processing. This should be the primary reference for the mass budget going forward.

3.2  Confirm panel construction and material

The six face panels and the shear panel collectively represent 75% of the estimated assembly mass. All were modelled here as solid aluminium (2700 kg/m³). If the panels are sandwich construction with CFRP face sheets and aluminium honeycomb core — which is typical for this class of spacecraft — the effective density is in the range of 600–900 kg/m³ for the panel as a whole. This correction alone could reduce the total mass estimate by 8–12 kg. The mechanical team should confirm the panel material specification and, if the CAD models are simplified solids rather than true sandwich representations, override the panel densities accordingly in the mass budget tool.

3.3  Verify COTS component masses against datasheets

Geometric volume is not a reliable mass estimator for COTS components, which are typically represented in assemblies as simplified or defeatured bodies. The following items in particular should be replaced with datasheet-quoted masses:

BHT-350 T1 (Busek Hall-effect thruster): The script returns 2.12 kg per unit from a geometry that may be a placeholder model. The Busek datasheet should be the authoritative source.

RS-00000821, RS-00000873, RS-00000450: Part numbers not resolved from the assembly name. Identify via the BOM and source from the supplier datasheet.

COTS-000093, COTS-000110, COTS-000111, COTS-000112, COTS-000114: Internal COTS designators. Cross-reference with the procurement register to obtain quoted masses.

3.4  Address duplicated part names in STEP export

SolidWorks AP214 exports are appending each part's name to itself (e.g. "Z-_Panel_-(2)_Z-_Panel_-(2)"). While this does not affect the volume computation, it complicates BOM readability and downstream tooling. The issue can be resolved by selecting AP242 format on export, or by adjusting the STEP export options under File → Save As → Options (uncheck "Export IDs").

3.5  Establish a controlled mass budget baseline

Once material assignments and COTS masses are confirmed, a formal mass budget baseline should be established with mass margins applied per the project's systems engineering margin policy (typically 20% at component level at this stage). The mass budget tool used here can be re-run against any updated STEP export and accepts a JSON override file for COTS and material corrections, allowing iterative updates without re-parsing the full assembly each time from scratch.